CEG 4430 Midterm Review

If IP packets with spoofed source IP addresses can be detected and discarded when they attempt to enter the Internet, then RST injection attack is impossible.

TRUE/FALSE?

True

Design one attack to gain access to the web service without knowing the username and password by exploiting the following PHP script.

DROP TABLE ‘login’;--

If a censor only sends out RST packets to clients, how can clients bypass RST injections?

Don’t accept RST packets

Introduction:

Assets are accessed only by authorized parties.

- Often referred to as privacy or secrecy.

Assets are accessible to authorized parties at appropriate times.

Vulnerability

-	A weakness in the security system that might be exploited to cause loss or harm.

Threat

-	A set of circumstances that has the potential to cause loss or harm.

Attack

-	The execution of a plan to carry out a threat by exploiting a vulnerability.

Network

- A network is a set of protocols, software, and hardware used to exchange information among different systems.

The Internet is a global-scale network

-	Offers general-purpose service

-	Has billions of users worldwide

-	Consists of millions of "smaller" networks

-	Supports numerous

types of services (Web, Email, VoIP...)

The Internet mainly uses the standard Internet protocol suite (a.k.a., TCP/IP).

The Internet Protocol (IP) address

-	Logical address

-	Manually or automatically assigned to a network interface controller (NIC)

-	32 bits for IPv4

Media Access Control (MAC)

address

-	Physical address

-	Assigned to a NIC by the manufacturer and are stored in hardware or firmware

-	48 bits

UDP offers best-effort, unreliable service — E.g., if a packet is dropped, UDP does not have mechanism to inform networked nodes.

TCP–Reliable–Relatively expensive

UDP–Best-effort–Inexpensive

Protocols that support functions based on the services provided by the transport layer

-	DNS

-	FTP -HTTP -IMAP -IRC

Tools

—	TCPDUMP/WireShark

—	Ngrep

Programming

— Writing your own packet sniffer

Libpcap

A collection of APIs for a program to capture packets from a network interface

Programming with Pcap

Existing tools are excellent

But we need more flexibility to develop customized analysis capabilities

—	Intrusion detection based on statistical analysis

-	Rich semantics in traffic analysis

• A packet that has never been seen for the last 7 days

Protocol/Vulnerability

The Data Link Layer

-	Above the physical-layer

—	Ethernet is one of the most popular ways for the data link layer

Ethernet:

Two typical implementations

-	10Base2

•	Each computer is attached to the cable using a T- Connector

—	Ethernet hub

•	Each computer is attached to the hub through an input/output port.

An Ethernet switch forwards a packet to the appropriate destination instead of broadcasting it.

What is ARP?

Host 1 wants to send a packet to host 3.

-	It knows the IP address of host 3

-	It does not know the MAC address of host 3

ARP Protocol

The Address Resolution Protocol (ARP)

— A link-layer protocol that provides services to the network layer

— Used to find a host's MAC address given its IP address

Solutions to ARP Spoofing

Checking for multiple occurrences of the MAC address

Static ARP tables

Maximum Transmission Unit

A network link is constrained by the size of the packet is can deliver Maximum Transmission Unit (MTU)

- The largest IP packet a link will accept

IP Fragmentation

If an IP packet is longer than the MTU of a link, the router breaks packet into smaller packets

—	Each smaller packet is a fragment

—	A fragment is still in the format of an IP packet

Fragmentation/Defragmentation

Fragmentation

-	The router usually performs the fragmentation

-	The sender/host can deliberately perform fragmentation

Defragmentation

-	The destination host usually defragments the fragmented packets

-	Use a timer to handle any lost fragment (i.e., discard the whole packet)

IP Fragmentation Attack

Denial-of-Service (DoS) Attack

• An attack used to cause a machine/service unable to perform its essential functionality

IP-Fragmentation-based DoS

—	1st fragment: flag = 001, offset = 0;

—	2nd fragment: flag = 001, offset = 64800

Consequence

—	the machine will allocate 64800 bytes memory, and hold it for some time (e.g., 15 to 255 seconds)

IP Spoofing

Sending an IP packet with forged source IP address

To impersonate another host with respect its IP address

Applications

-	Denial of Service Attacks

—	Access the IP-based authentication services such as the R services suite (rlogin, rsh, etc.)

SYN Flood Attack

Denial-of-Service (DoS) Attack

-	An attack used to cause a machine/service u to perform its essential functionality

SYN flood

-	A specific type of denial-of-service attack

-	Targeting at the implementation of the TCP protocol

A TCP reset basically kills a TCP connection instantly.

— If the RST bit is set to 1, it indicates to the receiving computer that the computer should immediately stop using the TCP connection; it should not send any more packets using the connection's identifying numbers, called ports, and discard any further packets it receives with headers indicating they belong to that connection.

Discussion

Why does GFW send RST packets to both client and server?

— GFW can send RST to only client. It should be enough to terminate the connection.

Race Condition

The sender can send a few (decided by both congestion control and flow control algorithms) packets with out receiving acknowledgments.

-Seq number < Min(window-size, rwnd)

•	Window-size for congestion control, decided by the sender of the packet

•	rwnd for flow control, decider by the receiver of the packet.

Detection Rules

RST_SEQ_DATA:

-	The RST packet is "out of sequence", with the receiver observing a sequence number less than the preceding data packet would suggest.

-	What doesn't it happen for normal cases?

DATA_SEQ_RST:

-	The receiver will see further data packets from the sender after it has already received the RST.

-	What doesn't it happen for normal cases?

Detection Rules

RST_SEQ_CHANGE:

-	Back-to-pack pairs of RSTs in which the second RST has a sequence number higher than the first, and that exceeds the current maximum sequence number.

—	What doesn't it happen for normal cases?

Detection Rules

RST_ACK_CHANGE:

-	Nonsensical ACK numbers

SYN_RST

-	SYNs immediately followed by RST

SYN_ACK_RST

-	SYN/ACKs immediately followed by RST

Domain Name System (DNS)

Indispensable component for the Internet

-	www.google.com -> 173.194.75.106

Used by a huge percentage of Internet applications

-	Browsers -FTP

-	Instant Messengers

Honey-Query

What do you expect after you send out the honey-query?

-	Not censored?

•	No DNS response at all.

—	Censored?

•	A DNS response

•	Is the IP address in the response trustable?

Do not forget

All honey queries are sent from real IP addresses controlled by authors

These IP addresses are unlikely to reside in the censored networks

Censors should actually not censor these IP addresses

But they inject fake DNS responses anyway - "the DNS injector does not consider packet origin when injecting packets".

Application Vulnerabilities

Two General Approaches

Identify the credential to break in

-	Weak password

-	Brute force

-	Social engineering

Compromise the program vulnerabilities

-	Incomplete mediation

-	Timing attacks

-	Buffer overflow

Buffer Overflow

A Buffer Overflow is an anomaly where data is stored beyond the boundary of a fixed-length buffer.

Program V.S. Process

Program

—	A file

•	Static instructions

•	Static data

Process

-	An instance of a program

•	Loaded into the memory

•	Executing and dynamic

HTTP Response

Data

-HTML

Executable Scripts

—	Javascript

-	VBScript

How Does a Server Generate Responses

Static Data

-	E.g., an pure HTML file (with javascript)

Scripts: dynamically generate content

—	PHP -ASP.NET -Java

SQL Injection Attack

Exploit a security vulnerability in an SQL application (e.g., web application)

- A specific case of "incomplete mediation"

Considered as one of the top 10 web application vulnerabilities of 07 and 10

A large number of incidents

-	2014: Biomedical Engineering Servers, Johns Hopkins University

—	2013: 71 Chinese government databases are compromised using SQL injection

Same-Origin Policy

One webpage can only read properties of another webpage if they share the same server, protocol, and port

If the same server hosts unrelated sites, scripts from one site can access document properties on the other

Two Types of Attacks

Persistent XXS Attacks

-	Saved persistently in the database

-	Damage massive users

Non-Persistent XXS Attacks

-	Targeted victims

TaintTracker

Tracks each instruction that manipulates data in order to determine whether the result is tainted.

—	Data movement instructions

•	LOAD, STORE, MOVE, PUSH, POP, and etc.

•	Rule: if any byte of data at the source location is tainted, the data at the destination will be tainted

-	Arithmetic instructions

•	ADD, SUB, XOR, and etc.

•	Rule: the result will be tainted if any byte of the operands is tainted

Randomization

Identify keywords in an SQL statement

Rewrite all keywords with the random key appended.

De-randomization

Identify terms/keywords (e.g., starting from the original keywords)

—	E.g., selectl23 or select456 or select Evaluate the format

—	Keyword + random number

—	Detect mal-formed keywords Stripping away the random number Send it to the DBMS

Parse Tree Validation

Objective

-	Detect SQL injection attacks at runtime.

Observation

-	All SQL injections alter the structure of the query intended by the programmer.

Web Security

Complexity

-	Protocols

•	Client

•	Server

-	Browsers

•	Data

•	Scripts

-	Servers

•	Data

•	Scripts

Example

Each file on the web server (under the web directory) can be invoked directly from the network by giving its name as part of the URL - -> Each file represents an unintended entry point

Detection

Signature-based detection

-"DROP Table"

-"Insert into"

Anomaly Detection

- Query syntax analysis

• Injected content change the parse tree of a query

Prevention

Mediate Input

- Use pattern matching

Hide error messages

READ ONLY Database Access

Prevention

Whitelisting

- Define what should be accepted

Blacklisting (NO)

— It can never be complete

Malicious Applications

Malware

Insider Attacks

-	Backdoors/trapdoors

-	Logic Bombs

Virus

-	Polymorphic and Metamorphic Virus Trojan Horses Privacy-Invasive Software

-	Adware/Spyware

Worm

Botnet

Backdoor/Trapdoor

Secret entry point into a system

-	Specific user identifier or password that circumvents normal security procedures

Commonly used by developers

—	Could be included in a compiler

Logic Bomb

A logic bomb is a class of malicious code that activated when a specific condition occurs.

—	Certain file is created

—	Certain time is reached

—	Certain user logins into the system

A logic bomb could represent

—	an "insider" threat

-	inherently malicious code

Virus

Self-replicating code

-	Malicious functions + self-replicating (with users' involvement)

-	"Normal Code" => "Normal Code + Malicious Code" Stealthy

-	Attempts to evade detection Operates when infected code executed

-	Propagates

-	Performs malicious actions

-	Redirect to the normal code

Virus Infection Vectors

Boot Sector

-	Loaded when the system is booted

Executable

-	Activated when an executable file is activated

Macro files

-	Triggered when a document is loaded

Virus Characteristics

Terminate and Stay Resident

-	Stays active in memory after application completes

-	Allow infection of previously unknown files

•	Traps calls that execute a program

Stealthy

-	Conceal Infection

•	Trap read and disinfect

•	Let executable call infected file

-	Polymorphic/Metamorphic Virus

•	Change virus code to circumvent misuse detection

Trojan Horse

A Trojan Horse is a malicious program that is disguised as legitimate software

The gift horse left outside the gate of Troy by the Greeks

-	Appear to be interesting and innocent

-	Actually harmful

Privacy-Invasive Software

Malware with specific malicious objectives

-	Adware

•	Pop up advertisements

-	Spyware

•	Key logging, screen capturing

Worms

Definition: Programs that self-propagate across the internet by exploiting security flaws in widely used services

Virus v.s. Worms

rus

A malicious program Propagates depending on the user intervention

•	File sharing

•	Execute an infected file

• Worm

-	A malicious program

-	Propagates automatously

•	Self-replicate

•	No user intervention is required

A Brief History of Worms

Worms that affect the operation of the whole Internet

-	Morris Worm (1988)

-	Code Red (2001)

-	Nimda (2001)

-	Blaster (2003)

-	SQL Slammer (2003)

Morris Worm

The positive impacts

-	Computer Emergency Response Team (CERT)

•	Reacted to the damage and disruption caused by Morris worm

•	Becomes a leading center on information sharing with respect to software vulnerability and malware

—	Raise attentions to cyber-security

The Code Red Worm

Rapid propagation

-	> 2,000 hosts/min

-	Code Red II Took about 14 hours to fully propagate

359,000 hosts are infected

Exploits

-	BOF in Microsoft IIS web server (enabled by default)

The Nimda Worm

Rapid Propagation

-	Became the Internet's most widespread worm within 22 minutes

Exploits

-	MS IIS vulnerability (CVE-2000-0884)

-	Email itself as attachment based on email addresses harvested from the infected machine

-	Copy itself across open network shares

-	Add exploit code to web pages on compromised servers in order to infect clients that browse the pages

-	Leverage the backdoors left behind by Code Red II.

Blaster Worm

Exploits

-	BOFin MS OS

-	The patch had been released one month earlier than the incident

• "The original Blaster was created after a Chinese cracking collective called Xfocus reverse engineered the original Microsoft patch that allowed for execution of the attack" from wiki

Attack

-	Launched DDoS attacks against windowsupdate.com

The Code Red I

Its Scanning Strategy

—	Launch 99 threads and each thread generated random IP addresses in the whole IP space (2A32)

—	Probe each IP to determine whether the vulnerable service is available on the host.

- If so, infect the host.

Hit-list scanning

How to collect hit-list in practice?

—	Stealthy scans

—	Distributed scanning

—	DNS searches

—	Spiders

• 33% of automated search engine queries are looking for vulnerable Internet services.

—	Public surveys -Just listen

Permutation Scanning

Random scanning

-	The Code Red I is a salient example to use random scanning

—	Disadvantage: many addresses are probed multiple times

Permutation Scanning

-	Objective: provide a self-coordinated, comprehensive scan while maintaining the benefits of random probing

—	Assumption: a worm can detect that a particular target is already infected

Permutation Scanning

All worms share a common pseudo random permutation of the IP address space

Any machine infected during the hit-list phase (or local subnet scanning) starts scanning after its point in the permutation

Whenever the worm sees an already infected machine, it chooses a new, random start point and proceeds from there

Worms infected by permutation scanning would start at a random point